This patient is for

- Lumbar/Thoracic/Cervical /Pelvis/Lumbo- Sacral plexus/Brachial Plexus

- Open/Closed

- With Sedation / Without Sedation

- With/ Without contrast

- Special technique for Artefact reduction from previous Hardware

General MRI guidelines:

- MRI should be of at least 1.5 Tesla or above.

- For Patients with hardware will prefer 1.5 Tesla

- Please use optimal TR/ TE, Field of View, Matrix, voxel size and slice thickness

for good resolution.

- Please take care to orient the cuts for axial, sagittal and coronal cuts.

- Make sure that Field of view covers the entire spinal element and beyond as described below

- We prefer closed MRI – if not possible – patient can be send to- wide bore Machine before going for – Open MRI.

Please provide us with following MRI sequences:

Cervical Spine

- Sag T2—FSE/TSE (Required)

  - The Field of View should be from Pons to T4 and from one CM Right to the transverse process to the one CM beyond the left transverse process of T1

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band in the neck in front of esophagus.

- Sag T1—FSE/TSE (Required)

  - The Field of View should be from Pons to T4 and from one CM Right to the transverse process to the one CM beyond the left transverse process of T1

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band in the neck in front of esophagus

- SAG STIR T2 (Required)

  - The Field of View should be from Pons to T4 and from one CM Right to the transverse process to the one CM beyond the left transverse process of T1

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band in the neck in front of esophagus

- Axial T2 FSE/TSE (Medic) (Required)

  - While taking the axial cuts please align the cuts to the disc

  - Please take at least 3 cuts through the disc area and 1 cut through the endplate – total 5 per block

  - Use at least 5 blocks

  - Do not put saturation band.

- Axial T1TSE (Required)

  - While taking the axial cuts please align the blocks to the disc both in sagittal and coronal plane

  - Please take at least 3 cuts through the disc area and 1 cut through the endplate – total 5 per block

  - Use at least 5 blocks

  - Do not put saturation band.

- Axial -Gradient Spin Echo images (Optional) (Required)

  - While taking the axial cuts please align the blocks to the disc both in sagittal and coronal plane

  - Please take at least 3 cuts through the disc area and 1 cut through the endplate – total 5 per block

  - Use at least 5 blocks

  - Do not put saturation band.

- Right Foraminal views (Optional) (Required)

  - 20 to 30 degrees oblique to coronal Sag T1

  - 3mm slices perpendicular to the neural foramen, set up on the axial T1 image at the level of C5/6 or C6/7.

- Left foraminal views (Optional) (Required)

  - 20 to 30 degrees oblique to coronal Sag T1

  - Sections to be acquired in the plane of the emerging nerve roots

- Coronal T2 image (Optional) (Required)

  - Field of view 1cm front of the vertebrae to 1 cm beyond the spinous process.

- Myelogram (Required)

  - 3-dimensional Myelogram.

  - Heavy T2 weighted with- MIP – FSE – Fat suppression

Lumbar Spine

- Sag T2 FSE/TSE (Optional) (Required)

  - The Field of View should be from T11 to coccyx and from one end of the Right transverse process to the other end of left transverse process.

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band on the abdomen in front of Aorta and blood vessels.

- Sag T1 FSE/TSE (Optional) (Required)

  - The Field of View should be from T11 to coccyx and from one end of the Right transverse process to the other end of left transverse process.

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band on the abdomen in front of Aorta and blood vessels.

- SAG STIR T2 (Optional) (Required)

  - The Field of View should be from T11 to coccyx and from one end of the Right transverse process to the other end of left transverse process.

  - The cuts should be parallel to a plane joining the center of the body and the center of the spinous process.

  - Position the block parallel to the spinal cord

  - Saturation band on the abdomen in front of Aorta and blood vessels.

- AX T2 FSE/TSE (Optional) (Required)

  - While taking the axial cuts please align the blocks to the disc both in sagittal and coronal plane

  - Please take at least 3 cuts through the disc area and 1 cut through the endplate – total 5 per block

  - Use at least 5 blocks

  - Do not put saturation band.

- Ax T1 FSE /TSE(Optional) (Required)

  - While taking the axial cuts please align the blocks to the disc both in sagittal and coronal plane

  - Please take at least 3 cuts through the disc area and 1 cut through the endplate – total 5 per block

  - Use at least 4 blocks

  - Do not put saturation band.

- Right Foraminal views (Optional) (Required)

  - 20 to 30 degrees oblique to coronal  Sag T1

  - 3mm slices perpendicular to the neural foramen, set up on the axial T1 image at the level of L3/L4 or L4/L5.

- Left foraminal views (Optional) (Required)

  - 20 to 30 degrees oblique to coronal  Sag T1

  - 3mm slices perpendicular to the neural foramen, set up on the axial T1 image at the level of L3/L4 or L4/L5.

- Coronal T2 image (Optional) (Required)

  - Field of view 1cm front of the vertebrae to 1 cm beyond the spinous process.

- Myelogram (Required)

  - 3-dimensional Myelogram.

  - Heavy T2 weighted  with- MIP – FSE – Fat suppression

- NEUROGRAPHY (Optional)

## Contrast MRI

- Sagittal T1 TSE   Fat saturated sequence with IV contrast.

- Axial T1 TSE Fat Saturated sequence with IV contrast.

## MRI on patients with metallic implants from previous spine surgery

- If Implants are there in the spine, please use appropriate sequences- appropriate sequences will not blur the image quality at implant site.

- Switch to TSE T2 for all levels of metal hardware with gradient echo sequence to minimize the influence of CSF pulsation.

- MR artifacts can be minimized by using newer techniques e.g. SEMAC-VAT and VAT techniques